RFID ATTENDANCE MANAGEMENT SYSTEM

Complete Technical Project Report & Comprehensive Documentation

PROJECT SUMMARY & EXECUTIVE BRIEF This project is an end-to-end IoT and Full-Stack Web Application for automated, fraud-proof classroom attendance management. It integrates physical RFID hardware (ESP8266 NodeMCU + RC522 reader) with a modern Node.js/Express backend, SQLite database, and a real-time React 19 frontend using WebSockets.

# 1. WHAT THE PROJECT DOES

The RFID Attendance Management System completely automates the process of recording and calculating student attendance in educational institutions. Traditional attendance systems rely on physical roll-call registers or manual paper sign-ins, which are time-consuming, prone to human error, and vulnerable to proxy attendance.

This project delivers a seamless digital workflow that bridges physical classroom presence with instant cloud synchronization:

- • Contactless Card Scanning: Instant Student Verification: When a student taps their contactless RFID card onto the hardware reader, their identity is verified within milliseconds.

- • Real-Time Screen Updates: Live Faculty Feed: As cards are tapped, the scanned student's name, college ID, department, and timestamp appear immediately on the teacher's active dashboard screen via WebSockets.

- • Anti-Fraud Mechanisms: Duplicate & Proxy Prevention: The system rejects cards tapped more than once within the same session and verifies valid card ownership.

- • Automated Absence Logic: Automated Absence Marking: Unscanned students are automatically marked "Absent" upon session finalization, eliminating manual roll-calling.

- • Eligibility Tracking: 75% Statutory Compliance Tracking: Automatically calculates overall and subject-wise percentages and flags students falling below the mandatory 75% attendance threshold.

- • Role-Based Access: Student & Faculty Portals: Provides dedicated, role-protected portals where students track attendance stats and faculty manage scanning sessions.

# 2. HOW THE SYSTEM WORKS (ARCHITECTURE & WORKFLOW)

The system follows a multi-tiered architecture combining IoT Hardware, RESTful APIs, WebSocket event streaming, Relational Database management, and Reactive Single Page Applications.

## 2.1 End-to-End Operational Lifecycle

- Step 1: Session Initialization — The teacher logs in to the Faculty Portal (e.g., MIS, IRS, STQA) and clicks "Start RFID Scan". The server initializes an attendance session in the database with status = "scanning".

- Step 2: Physical Hardware Tap — The student taps their 13.56 MHz Mifare RFID card on the MFRC522 scanner. The ESP8266 NodeMCU reads the Unique Identifier (UID) over the SPI bus.

- Step 3: Network Transmission — The ESP8266 transmits an HTTP POST request containing the UID to http://<SERVER_IP>:5000/api/rfid/scan over local Wi-Fi.

- Step 4: Backend Processing & Verification — The backend normalizes the UID, checks if a session is currently active, looks up the registered student in SQLite, and checks for prior scans in this session.

- Step 5: Real-Time Event Broadcast — The backend inserts a record into the temporary "attendance_entries" table and broadcasts a "studentScanned" event through Socket.IO.

- Step 6: Real-time UI Update — The teacher's browser receives the WebSocket event and immediately updates the live attendee list with a pulsing indicator and sound effect.

- Step 7: Session Finalization — When the lecture finishes, the teacher clicks "Save Attendance". The server converts all scanned entries into permanent "Present" records and marks all non-scanned enrolled students as "Absent".

- Step 8: Student Analytics Update — When students log in, the summary API aggregates total classes conducted, total classes attended, and dynamically calculates attendance percentages with visual color-coded badges.

## 2.2 Attendance Calculation & 75% Rule Engine

Every completed attendance session increments the total class counter for that subject. The attendance percentage calculation is strictly deterministic:

- Subject Formula: Subject Attendance (%) = (Present Sessions / Total Sessions Conducted in Subject) × 100

- Overall Formula: Overall Attendance (%) = (Total Present Across All Subjects / Total Classes Conducted Across All Subjects) × 100

- Threshold Rule: If Overall or Subject Attendance < 75%, the UI displays a danger alert (⚠️ Attendance Shortage Warning) informing the student that they are at risk of exam debarment.

# 3. WHAT IT USES TO DO IT (TECH STACK & HARDWARE)

The system is built entirely on modern, reliable, and lightweight industry-standard technologies without unnecessary bloat.

## 3.1 Hardware Components

| Component | Specification / Model | Role in the System |
| --- | --- | --- |
| Microcontroller | ESP8266 NodeMCU v3 (Wi-Fi Enabled) | Reads RFID data via SPI, connects to Wi-Fi, sends HTTP POST requests to backend. |
| RFID Reader | MFRC522 13.56 MHz RFID Module | Contactless reader utilizing electromagnetic induction to read 1K Mifare cards/keyfobs. |
| RFID Tags / Cards | Mifare Classic 1K RFID Cards (13.56 MHz) | Assigned to individual students; contains unique factory hardcoded UID. |
| Power & Interfacing | Micro-USB Cable & Jumper Wires | Powers the ESP8266 and connects hardware SPI pins (SDA, SCK, MOSI, MISO, RST). |

### Hardware Pinout / Circuit Connection Diagram

| RC522 Pin | ESP8266 NodeMCU Pin | Function / Description |
| --- | --- | --- |
| SDA (SS) | D4 (GPIO 2) or D8 (GPIO 15) | SPI Chip Select / Slave Select line |
| SCK | D5 (GPIO 14) | SPI Serial Clock line |
| MOSI | D7 (GPIO 13) | SPI Master Out Slave In (Data transmission) |
| MISO | D6 (GPIO 12) | SPI Master In Slave Out (Data reception) |
| RST | D3 (GPIO 0) | Module Reset pin |
| 3.3V | 3.3V | Power supply (3.3V DC only — 5V can damage RC522) |
| GND | GND | Common Ground |

## 3.2 Software Technologies & Frameworks

| Layer | Technology Used | Purpose & Key Capabilities |
| --- | --- | --- |
| Frontend Framework | React 19 + Vite 6 | Modern, ultra-fast single page reactive UI with client-side routing. |
| Backend Runtime | Node.js + Express.js | High-performance asynchronous REST API server handling authentication, RFID scans, and sessions. |
| Database | SQLite3 (sqlite3 driver) | Lightweight, self-contained, zero-configuration relational database with foreign key support. |
| Real-Time WebSockets | Socket.IO (v4.8) | Bi-directional low-latency event communication between hardware scan endpoint and teacher UI. |
| Security & Auth | JWT (JSON Web Tokens) & BCryptJS | Secure password hashing with salted rounds and stateless token-based authentication. |
| Embedded Firmware | Arduino C++ (ESP8266HTTPClient + MFRC522) | Embedded firmware running on NodeMCU to poll RFID cards and dispatch HTTP requests. |
| Styling System | Pure Vanilla CSS & Glassmorphism Design | Custom CSS variables, sleek dark and light cards, responsive grids, and micro-animations. |

# 4. DATABASE ARCHITECTURE & SCHEMA DESIGN

The relational database schema is designed to ensure strict data integrity, fast querying, and historical auditability.

| Table Name | Key Fields | Description & Relationship |
| --- | --- | --- |
| users | id, email, password_hash, role | Stores login credentials for students and teachers. Password secured via bcrypt. |
| subjects | id, name | Curriculum subjects (e.g., MIS, IRS, STQA). |
| students | id, user_id, college_id, name, department | Student demographic details, linked to users via user_id foreign key. |
| teachers | id, user_id, name, subject_id | Teacher profiles linked to their designated subject via subject_id. |
| rfid_cards | id, uid, student_id | Maps physical card UID hex strings to student records (UNIQUE constraint on UID). |
| attendance_sessions | id, teacher_id, subject_id, session_date, status | Tracks live and past scanning sessions (status: scanning, completed, cancelled). |
| attendance_entries | id, session_id, student_id, scanned_at | Temporary live scan entries while a session is active. Prevents double scans. |
| attendance_records | id, session_id, student_id, subject_id, date, status | Finalized permanent record for each student in every class session (Present/Absent). |

# 5. REST API & WEBSOCKET EVENT SPECIFICATION

| Endpoint | Method | Access Level | Description |
| --- | --- | --- | --- |
| /api/auth/login | POST | Public | Authenticates student or teacher; returns JWT token and user profile. |
| /api/auth/me | GET | Bearer Token | Returns the profile of the currently logged-in user. |
| /api/students/me/attendance/summary | GET | Student Only | Returns total classes, present count, overall %, and subject-wise breakdown. |
| /api/students/me/attendance | GET | Student Only | Returns complete date-wise attendance history logs. |
| /api/teachers/me | GET | Teacher Only | Fetches teacher subject info, active session status, and past session history. |
| /api/attendance/start | POST | Teacher Only | Starts or resumes an active RFID scanning session for teacher's subject. |
| /api/attendance/session/:id | GET | Teacher Only | Retrieves session metadata and live list of scanned students. |
| /api/attendance/session/:id/cancel/:entryId | POST | Teacher Only | Cancels an erroneous scan entry in the active session. |
| /api/attendance/session/:id/save | POST | Teacher Only | Finalizes session, writes Present/Absent records, and closes scanning. |
| /api/rfid/scan | POST / ALL | Hardware / Simulator | Accepts RFID card UID, verifies student, saves temporary scan, and triggers WebSocket broadcast. |

## 5.1 Real-Time WebSocket Events (Socket.IO)

- • studentScanned: Emitted when ESP8266 scans a card. Transmits student name, college ID, department, and scan timestamp to live teacher radar screen.

- • entryCancelled: Emitted when teacher clicks cancel on a scanned student entry. Removes the entry from all connected teacher viewports.

- • sessionSaved: Emitted when teacher saves attendance. Updates session status to completed and displays confirmation.

- • attendanceUpdated: Broadcasted to all student connections so student dashboards automatically refresh their summary percentages.

# 6. CONCLUSION & FUTURE SCOPE

The RFID Attendance Management System represents a robust, highly scalable, and production-ready academic IoT solution. By combining cost-effective microcontrollers with high-speed WebSockets and reactive interfaces, it eliminates classroom proxy attendance, provides instant feedback to faculty, and keeps students transparently informed about their attendance compliance.

Future enhancements can include SMS/Email alerts to parents when a student falls below 75%, biometric multi-factor authentication, and automated timetable scheduling integration.